Hollow retroreflectors

1. Advantages

· Lighter than solid cubes

· Transparent at all wavelengths

· No transmission though glass that may 	have thermal gradients

· No polarization effects

· High gain by using a single dihedral angle 	offset

2. Disadvantages

· Smaller acceptance angle

· Low range of velocity aberration if 	narrow return beam is used

3. Potential problems

· Joints may be unstable under thermal 	stress

· Thermal gradients can warp the surfaces 	or change the dihedral angles

4. Types of retroreflectors

· Solid block of material (no such thing)

· Beryllium with epoxy joints

· Aluminum with bolted joints

· Coated quartz reflecting plates

5. Potential applications

· GPS

· GALILEO

· Geosynchronous

· Lunar

The incidence angle on the array does not vary much so the small acceptance angle is not a problem. In low earth orbit more cubes would be needed to cover the large range of incidence angles.

6. Existing hollow cubes in space

· RME (6” cube tracked by 48” telescope)

· ADEOS (.5 meter coated quartz plates -  	tracked by the network)

· TES (inside instrument at -90C.)

7. Thermal modeling

A. Design tolerances.

Since there is no  quartz, the performance of the cube depends only on maintaining the mechanical tolerances to l/10. The main problem is solar heating of the cube.

B. Absolute temperature.

Metals run hot because they have a high a/e ratio. This may damage the epoxy used to join the reflecting surfaces. Any thermal control needs to be passive through the use of materials with low absorptivity and high emissivity.

C. Thermal gradients

Suppose we have a square plate of area l x l and thickness w. The thermal parameters are

a = solar absorptivity

= emissivity of the front surface

= emissivity of the back surface

S = solar constant = 1412.5 Watts/sq meter

k = thermal conductivity of Beryllium = 	225 Watts/m-°K

c = linear expansion coefficient of 	Beryllium =

s = Stefan Boltzman constant =

f  = fraction of the solar radiation conducted 	through plate.

Parametric equations have been derived for estimating the mechanical distortion due to thermal gradients in the cube as a function of the mechanical and thermal parameters.

Conduction through a plate.

All plates are being equally heated. The heat is being conducted through the plate and radiated from the back surfaces.

If one side of the plate is warmer than the other the plate would warp into a spherical cap where d is the deflection.

Conduction along a plate.

The solar heating is only on one plate. Heat is being conducted to the other plates that are not being heated significantly.

The change in linear dimensions d could distort the dihedral angles.

The equations show that thermal warping is proportional to the square of the size of the cube corner for both types of thermal gradients.

Calculations using the formulas above indicate the thermal warping of a cube in the size range 1.5 to 2.0 inches is acceptable. The unresolved issue is the stability of the joints under thermal stress.

More detailed engineering studies are now being done at NASA to evaluate the mechanical distortion and possible damage to the joints due to thermal problems.

The best design would be a single block of material. Unfortunately, there is no existing technology for polishing the inside surfaces.

The next best choice would be a reflector with no dissimilar materials to cause differential expansion and contraction.

Composite cubes have been successfully used under thermally controlled conditions.

8. Conclusions

The use of hollow cubes for laser ranging appears to be feasible if the issue of the stability of the joints can be resolved.